# 企業資訊通訊與網路　新版期末考古題：題目與解答

本文件收錄兩份較新的期末考卷（2025、2023），題目＋參考解答。解答依課本投影片 ch06–ch11 內容整理（藍字），並標註課本依據。

與舊三卷（109-1、2020、卷三）考點重複率約 82%；新增考點（IPv4 header length、T1 bonding、Service Band/Channels、SMTP/POP/IMAP 解釋、欄位細項等）已一併納入。

# 試卷一：企業資訊通訊與網路 期末考 (2025)

## A. 填充 (Fill in the Blanks) (39%)

1. "fe80:1::a3:b40:8" is the simplified representation of an IPv6 address. The original IPv6 address is ____.

【解答】fe80:0001:0000:0000:0000:00a3:0b40:0008（現有 5 段，:: 代表 3 段全零；每段補滿 4 位）。　〔ch08 p77–82〕

2. 123.25.174.88 bitwise-AND 255.255.240.0 = ____.

【解答】123.25.160.0（174 & 240 = 10101110 & 11110000 = 10100000 = 160；第 4 octet & 0 = 0）。　〔ch08 p19–21,55–58〕

3. By inverse square law attenuation, the signal strength at 2 meters is 40 mW. The signal strength at 8 meters would be ____ mW.

【解答】2.5 mW。S2 = 40 × (2/8)² = 40 × 1/16 = 2.5。　〔ch06 p14–16〕

4. The value of the header length field in an IPv4 header is ____ if the length of the IPv4 header is 20 octets.

【解答】5（header length 以 4 octets 為單位：1 = 4 octets，5 = 20 octets）。　〔ch08 p68〕

5. When an IPv6 packet is forwarded by a router, the ____ header field of the packet will be decreased by one.

【解答】Hop Limit（功能等同 IPv4 的 TTL）。　〔ch08 p88〕

6. Transport mode and Tunnel mode are two modes of operation in IPsec VPN. In the ____ mode, only the two IPsec gateways need digital certificates.

【解答】Tunnel（隧道模式；只有兩端 gateway 需憑證與設定）。　〔ch09 p50–52〕

7. ____ provides residential Internet access over one-pair voice-grade UTPs and offers high downstream speeds but limited upstream speed.

【解答】ADSL（Asymmetric DSL；利用既有 1-pair VG UTP，下行高上行低）。　〔ch10 p12〕

8. 802.11 WLANs use 2.4GHz and 5GHz unlicensed bands. Which provides more bandwidth? ____

【解答】5 GHz（頻寬較多，可有 11–24 個非重疊通道；2.4 GHz 只有 3 個）。　〔ch06 p37〕

9. 802.11i provides two modes of operation. In the ____ mode, all hosts use the same key (password) to access an access point.

【解答】PSK（Pre-Shared Key；所有站台共用同一把預共享金鑰）。　〔ch07 p13〕

10. There are Wi-Fi, Bluetooth, NFC, and RFID wireless technologies. Which provides short distance communication within 10 meters? ____

【解答】Bluetooth（PAN，最大距離約 10 m）。〔註：NFC 更短，僅約 4 cm；若題意指「恰約 10 公尺」則為 Bluetooth〕　〔ch07 p50–52〕

11. ____ is a routing mechanism in WANs. A label is inserted in front of the IP header. Routing is performed via the label.

【解答】MPLS（Multiprotocol Label Switching；以 label 查表轉送，較傳統路由便宜）。　〔ch10 p52–56〕

12. Assume there are three solutions of leased lines: T1, five bonded T1, and T3. Which is the fastest? ____

【解答】T3（44.736 Mbps）。T1 = 1.544 Mbps、五條 bonded T1 = 5 × 1.544 = 7.72 Mbps，皆小於 T3。　〔ch10 p41,44〕

13. ____ allows a number of hosts with private IP address to access the Internet via the same public IP address.

【解答】NAT（Network Address Translation；多台內部私有 IP 共用對外公有 IP）。　〔ch09 p16–25〕

## B. 簡答 (Short Answer)

1. EMI, Dead(shadow) zones, Inverse square law attenuation, Absorptive attenuation, and Multipath Interference are the five wireless propagation problems. (9%)

(1). Which is the most serious propagation problem?

(2). Which two problems get worse as frequency increases?

【解答】(1) Multipath Interference（多路徑干擾；WLAN 頻段最嚴重）。(2) Dead/shadow zones（死區）與 Absorptive attenuation（吸收衰減）隨頻率升高而變嚴重。　〔ch06 p17–22〕

2. CSMA/CA+ACK and RTS/CTS are the two MAC scheme used by 802.11 WLANs. (8%)

(1). In CSMA/CA+ACK, CA means Collision Avoidance. How can collisions be avoided possibly?

(2). Suppose a host is to send a frame to access point AP1 via RTS/CTS. How does the RTS/CTS scheme work?

【解答】(1) 當前發送者停止後，等待中的各裝置先各自等一段「隨機」退避時間再傳，避免同時搶送造成碰撞。(2) 傳送前先送 RTS、AP 回 CTS，所有聽到 CTS 的站台都先暫停傳送，藉此解決「隱藏節點」問題。　〔ch06 p59–61,69〕

3. (1). Describe the three steps of opening a TCP connection. (8%)

(2). Describe the four steps of closing a TCP connection.

【解答】(1) 三次握手：SYN → SYN/ACK → ACK（同步雙方初始序號後建立連線）。(2) 四次揮手：FIN → ACK → FIN → ACK（TCP 全雙工，兩個方向各自關閉）。　〔ch08 p104,109〕

4. Suppose the network part of the IPv4 addresses assigned to company ABC is 16 bits. Company ABC further uses 5 bits as the subnet part of the IP addresses. (6%)

(1). How many subnets does company ABC have at most?

(2). How many hosts does each subnet have at most?

【解答】(1) 2⁵ − 2 = 30 個子網。(2) host 部分 = 32 − 16 − 5 = 11 bits → 2¹¹ − 2 = 2,046 台。　〔ch09 p5–8〕

5. The following table shows a part of a routing table. Suppose the metric is cost. (12%)

（路由表共 10 列：Row1 0.0.0.0/0 cost80；Row2 123.0.0.0/8 cost60；Row3 123.45.0.0/16 cost40；Row4 123.45.0.0/16 cost35；Row5 123.45.1.0/24 cost20；Row6 123.50.2.0/24 cost10；Row7 123.60.5.0/24 cost30；Row8 123.60.5.0/24 cost25；Row9 123.80.7.0/24 cost10；Row10 123.80.8.0/24 cost30）

(1). Which row(s) are matched and which row will be selected for destination address = 123.45.1.21?

【解答】符合 Row 1,2,3,4,5；最長前綴 Row 5 為 /24（唯一）→ 選 Row 5。　〔ch08 p34–48〕

(2). Which row(s) are matched and which row will be selected for destination address = 123.20.5.11?

【解答】符合 Row 1,2；最長前綴 Row 2 為 /8 → 選 Row 2。　〔ch08 p34–48〕

(3). Which row(s) are matched and which row will be selected for destination address = 123.80.12.9?

【解答】符合 Row 1,2（.7/.8 子網皆 ≠ .12，Row 9/10 不符）；最長前綴 Row 2 為 /8 → 選 Row 2。　〔ch08 p34–48〕

6. Use the following SMTP commands to explain how a mail is sent via SMTP: . (dot), DATA, HELO, MAIL FROM, QUIT, RCPT TO. (8%)

【解答】順序：HELO（打招呼）→ MAIL FROM:（指定寄件者）→ RCPT TO:（指定收件者）→ DATA（宣告開始送內文）→（送出內文）→ .(單獨一個句點，即 <CRLF>.<CRLF>，結束訊息)→ QUIT（結束連線）。　〔ch11 p19–26〕

7. Choose one or more answers from the following pool of protocols. (3% per correct answer)

(1). Protocol, usually used by routers, to report possible errors during routing.

【解答】ICMP。　〔ch09 p69–71〕

(2). Protocol, mostly used by routers, to find the data link layer address of a host given its IP address.

【解答】ARP。　〔ch08 p60–64〕

(3). Signaling protocol for voice over IP.

【解答】SIP。　〔ch11 p41–42〕

(4). Protocol with a blank line (CRLF) between header and content (entity).

【解答】HTTP（標頭與 entity 之間有一空行 [CRLF]）。〔SMTP 亦有 header/body 空行，但「entity」為 HTTP 用語〕　〔ch11 p51–52〕

(5). Dynamic routing protocol.

【解答】OSPF（或 RIP / EIGRP / BGP 擇一）。　〔ch09 p62–68〕

(6). Protocol in the transport layer.

【解答】TCP 與 UDP（RTP 亦在傳輸之上加序號/時間戳）。　〔ch08 p98–117〕

(7). Protocol with a "Flow Label" header field for special handling of packets.

【解答】IPv6（Flow Label 為 IPv6 標頭欄位）。　〔ch08 p83–86〕

(8). Protocol with a "protocol" header field to indicate the upper protocol over the protocol.

【解答】IPv4（Protocol 欄位：1=ICMP、6=TCP、17=UDP）。　〔ch08 p67,71〕

(9). Protocol used by ping for measuring round-trip time.

【解答】ICMP。　〔ch09 p69–72〕

# 試卷二：企業資訊通訊與網路 期末考 (2023)

## A. 填充 (Fill in the Blanks)

1. "fe80::c2f:80a4:d0" is the simplified representation of IPv6 address: ____.

【解答】fe80:0000:0000:0000:0000:0c2f:80a4:00d0（現有 4 段，:: 代表 4 段全零）。　〔ch08 p77–82〕

2. 163.22.120.117 bitwise-AND 255.255.224.0 = ____.

【解答】163.22.96.0（120 & 224 = 01111000 & 11100000 = 01100000 = 96；第 4 octet & 0 = 0）。　〔ch08 p19–21,55–58〕

3. By inverse square law attenuation, the signal strength at 5 meters is 80 mW. The signal strength at 20 meters would be ____ mW.

【解答】5 mW。S2 = 80 × (5/20)² = 80 × 1/16 = 5。　〔ch06 p14–16〕

4. ____ access point impersonates as a legitimate one (e.g. an AP with SSID "NCNU"), usually operating at very high power, to attract wireless clients.

【解答】Evil twin（惡意雙生 AP；高功率假冒真 AP 做中間人攻擊）。　〔ch07 p24,32–33〕

5. In the ____ mode, one of two 802.11i modes, all wireless clients get the same password to access Wi-Fi.

【解答】PSK（Pre-Shared Key 預共享金鑰模式）。　〔ch07 p13〕

6. When an IPv4 packet is forwarded by a router, the ____ field in the IPv4 header will be decreased by one.

【解答】TTL（Time to Live）。　〔ch08 p70〕

7. If 4 bits are used as the subnet part of an IPv4 address, there can be ____ subnets allocated at most.

【解答】2⁴ − 2 = 14 個子網（扣全 0 與全 1）。　〔ch09 p6〕

8. ____ is a protocol usually used by routers to send error messages to hosts, e.g. host unreachable.

【解答】ICMP（Internet Control Message Protocol）。　〔ch09 p69–71〕

9. IPv6 has many optional headers. Each header is linked to the next one by the ____ field in each header.

【解答】Next Header（以此欄位串接延伸標頭；TCP=6、UDP=17、ICMPv6=58）。　〔ch08 p93–97〕

10. SYN, ____, and ACK are the three steps in opening a TCP connection.

【解答】SYN/ACK（三次握手：SYN → SYN/ACK → ACK）。　〔ch08 p104〕

11. ____ and RTP are the signaling and transport protocols of VoIP respectively.

【解答】SIP（Session Initiation Protocol，信令）；RTP 為傳輸。　〔ch11 p41–44〕

12. ____ is a routing mechanism in PSDN. It inserts a label in front of the IP header. Routing is based on the label.

【解答】MPLS（Multiprotocol Label Switching）。　〔ch10 p52–56〕

13. ____ provides residential Internet access over one-pair voice-grade UTPs and offers high downstream speeds but limited upstream speed.

【解答】ADSL（Asymmetric DSL）。　〔ch10 p12〕

14. Give an example of leased lines. ____ (T1/T3/E1/E3 擇一)

【解答】T1（1.544 Mbps；其他可接受：T3 44.736 Mbps、E1 2.048 Mbps、E3 34.368 Mbps）。　〔ch10 p41〕

## B. 簡答 (Short Answer)

1. Explain the following terms in wireless communication.

(1). Service Band and Channels

【解答】Service Band（服務頻帶）是一段特定的頻率範圍（如 2.4 GHz 帶）；Channel（通道）是在該 band 內再細分的特定頻率範圍（如 2.4 GHz 帶的 1/6/11 通道）。　〔ch06 p31,34〕

(2). Spread Spectrum Transmission

【解答】展頻傳輸：把訊號散佈到比所需更寬的頻寬，以提高抗干擾能力（尤其降低多路徑干擾）；2.4/5 GHz 頻段法規要求使用。　〔ch06 p45〕

(3). Channel Reuse (for Cellular Telephone Service)

【解答】頻道重用：在蜂窩網路中，於不同 cell 重複使用相同頻道，使營運商能服務更多客戶（例 500 頻道 × 100 cell = 50,000 同時用戶）。　〔ch10 p24–25〕

2. Compare Bluetooth and NFC/RFID in terms of applications, communication distances, and speeds. (可能會考)

【解答】Bluetooth：屬 PAN，有 Profiles（規範各應用如何協同，Wi-Fi 沒有）；距離較遠（約 10 m，高速版 ~30 m）、速度較快（經典 ~3 Mbps、高速 ~24 Mbps），用於耳機、滑鼠等周邊。NFC：極短距（≤4 cm）、低速（≤424 kbps），用於行動支付、門禁、交換名片。RFID：被動標籤無電源、靠掃描電力回應，便宜但資訊量小、不能加密，目標取代條碼。　〔ch07 p50–52,63–66〕

3. EMI, Dead(shadow) zones, Inverse square law attenuation, Absorptive attenuation, and Multipath Interference are the five wireless propagation problems. (9%)

(1). What is Multipath Interference?

(2). Which two problems get worse as frequency increases?

【解答】(1) 多路徑干擾：直接波與反射波同時到達接收端互相干擾；若兩波反相會抵消成無訊號（WLAN 頻段最嚴重）。(2) Dead/shadow zones（死區）與 Absorptive attenuation（吸收衰減）隨頻率升高而變嚴重。　〔ch06 p17–22〕

4. CSMA/CA+ACK is the MAC scheme used by 802.11 WLANs.

(1). What does "Carrier Sense Multiple Access" mean?

(2). CA means Collision Avoidance. How can collisions be avoided?

【解答】(1) 傳送前先「聆聽載波」，若偵測到別人正在傳就等待，藉此控制多台裝置不同時傳送。(2) 當前發送者停止後，各等待裝置先各自等一段「隨機」退避時間再傳，避免同時搶送造成碰撞。　〔ch06 p59–60〕

5. Explain the following header fields.

(1). Sequence Number / Acknowledgment Number (in TCP)

【解答】Sequence Number：每個 TCP segment 的序號，接收端用來重組成原訊息、偵測遺失；Acknowledgment Number：接收端正確收到後回送的確認段，指出在確認哪一段。　〔ch08 p100〕

(2). Source Port / Destination Port (in TCP)

【解答】Source Port：來源端的連接埠號；Destination Port：目的端的連接埠號（多工伺服器用 port 區分不同應用）。　〔ch08 p116〕

(3). Identification / Flags / Fragment Offset (in IPv4)

【解答】用於 IP 封包的分段（fragmentation）與重組：Identification 標記同一原始封包、Flags 控制是否可分段/是否還有後續、Fragment Offset 指出此片段在原資料中的位移。　〔ch08 p67–69〕

6. SMTP, POP/IMAP, and RFC 822/2822 are three parts of an E-mail standard. Please explain them shortly.

【解答】SMTP：簡單郵件傳輸協定，用於在伺服器間「寄送」郵件（近乎即時）。POP/IMAP：收件者「下載／收取」郵件的協定——IMAP 會把電腦與主機上的郵件同步，POP 則把郵件下載到本機。RFC 822/2822：規定電子郵件「格式」的文件標準（純文字訊息）。　〔ch11 p13–16〕

7. The following table shows a part of a routing table. Suppose the metric is cost.

(1). Which row(s) are matched? Which row will be selected for destination 140.121.33.10?

【解答】符合 Row 1, 6, 8, 9, 11；依最長前綴（平手再比 cost 取小）→ 選 Row 8。　〔ch08 p34–48〕

(2). Which row(s) are matched? Which row will be selected for destination 140.110.45.21?

【解答】符合 Row 1, 4, 5, 11；→ 選 Row 5。　〔ch08 p34–48〕

(3). Which row(s) are matched? Which row will be selected for destination 140.22.1.2?

【解答】符合 Row 1, 2, 3, 11；→ 選 Row 2。　〔ch08 p34–48〕

（2023 試卷未附完整路由表，以上為原卷給定之符合列與選定列；判斷方法同 2025 第 5 題：最長前綴優先，平手 cost 取小。）

8. What are (a) and (b) in the HTTP request message to get web page "http://abc.com/news/a.html"?

GET (a)____ HTTP/1.1 ；　Host: (b)____ ；　Connection: keep-alive

【解答】(a) = /news/a.html（GET /news/a.html HTTP/1.1）；(b) = abc.com（Host: abc.com）。　〔ch11 p51〕

9. Transport mode and Tunnel mode are the operating modes of IPsec. Explain the two modes.

【解答】Transport mode：建立完整的「主機到主機」端到端保護（Internet 與站內都保護），但每台主機都需憑證與設定，成本較高。Tunnel mode：只在兩端 IPsec gateway 之間建立保護（只護 Internet 段、站內不保護），只有 gateway 需憑證，成本較低。結論：安全 Transport 勝、成本 Tunnel 省。　〔ch09 p48–52〕